Oasis 3D Printer

Operations Report & Standard Operating Procedure (SOP)

Document Version: 1.1.6  |  Date: 2026-05-20

Software: Oasis_printer_v1.1.6.py + Study_params_loader_and_print.py

# 1. System Overview

## 1.1 Oasis_printer_v1.1.6.py

A PyQt5-based GUI application that simultaneously controls an HP45 inkjet printhead and a GRBL motion controller, implementing Binder Jetting 3D printing onto ceramic powder layers.

| Component | Description |
| --- | --- |
| MainWindow | Main GUI. Split into self.ui (separate window from .ui file) and self.form (widget references) |
| CameraController | Per-layer image capture (separate widget) |
| GRBL | Motion serial communication (XY + Z/A rollers) |
| HP45 | Inkjet printhead serial communication |
| ImageConverter | SVG/image → print array conversion |

Image for MainWindow.UI

Camera setting configuration. UI

## 1.2 Study_params_loader_and_print.py

An experiment automation script that reads parameters from oasis_change_parameters.csv and automatically injects them into the Oasis GUI. Guarantees a reproducible parameter set per run and saves execution records to the runs/ directory.

# 2. SOP — Standard Operating Procedure

## 2.1 Pre-Run Checklist

| Step | Check Item |
| --- | --- |
| 1 | Review parameter values in oasis_change_parameters.csv (especially variance entries) |
| 2 | Confirm GRBL motion controller COM port (Device Manager) |
| 3 | Confirm HP45 inkjet controller COM port |
| 4 | Visually inspect powder fill level and printhead condition |

Note : Additionally, the system must monitor the powder reservoir levels (should be around 70mm below) to ensure adequate supply and confirm that the build substrate is cleared of any previously fabricated structures

Command : you need to type on the terminal

- Cd Oasis_3DPrinter

- venv\Script\activate.ps1

- Python Oasis_printer_v1.1.3.py

## 2.2 GUI Startup Sequence

⚠️  The following sequence must be followed.

| Step | Action | Detail |
| --- | --- | --- |
| 1 | Load SVG File (First Priority) | File control panel - Open File → Select the target SVG file. - Need to check the maximum height will be fit in the range (middle of the bottom UI) Select the svg file you want to print out Check the printing part size in the middle bottom of UI |
| 2 | Connect Motor COM Port | Motion Control Panel → Select COM port (for Zwick laptop, COM3) → Click Connect |
| 3 | Run Homing | Click Home button — confirm machine returns to origin position before moving the part (there will be error message if you try to move others before click Home button) |
| 4 | Connect Printhead COM Port | Inkjet Control Panel → Select COM port (for Zwick laptop, COM4) → Click Connect |
| 5 | Verify Temperature Reading | Confirm T (temperature) value appears in the bottom-right of the information panel — indicates successful printhead connection |
| 6 | Test printhead working | Click Test head and see 300/300 (meaning all 300 point works okay) |
| 7 | Printhead cleaning | Cleaning Procedure (if poor jetting is observed) 1) Pink solution sprayed onto fine-cleaner 2) Softly remove clogged binder at the surface of the prinhead 3) Reinstall the printhead 4) Click “HeadClean” in UI and wait for 40s 5) See the binder is jetting when click “Prime” and re-verify jetting performance |
| 8 | Camera setting | Camera Configuration: 1) Open the Camera Panel and select Enable Camera. 2) Set the camera to Index 1 (Index 0 uses the integrated webcam for Zwick laptop). 3) Set the capture interval to 0.50 s or 1.00 s. 4) Define the Save Directory on the hardware drive (Organize folders by “Date_SampleName”). NOTE: Always re-verify camera settings when resuming after a printing error. 5) Run calibration |
| 10 | Start Print | Click Print (everything is setting up ), and verify stable processing |

Cleaning Sequence The sequence executes automatically with a 1000 ms interval between each burst:

Preheat x10 prime x10 Preheat x10 prime x5 preheat x5  Total 40burst (40seconds)

Important Notes for Cleaning

- Ensure the inkjet is connected (Connect button shows "Disconnect").

- Ensure the printer is not currently printing.

- The button is silently blocked if the inkjet is not connected or if a print is in progress.

- The cleaning sequence runs in a background thread (daemon=True) and does not block the UI.

- Preheat pulse count and Prime pulse count follow the values set in oasis_change_parameters.csv (preheat_pulses, prime_pulses). Defaults: 5000 / 100.

- It is recommended to run HeadClean before the first print of the day or after the printhead has been idle for an extended period.

2.4 Start-Layer Control (Mid-Print Resume)

If the printer stopped in the middle of the layer  restart the current layer for the next print run

1. Start-Layer Control (Mid-Print Resume)

Overview A new UI control has been added to allow printing to begin from a specified layer rather than always starting from Layer 1. This is useful when resuming a failed or interrupted print job.

Location in UI Inkjet / File Control panel → "Start from layer" spinbox (above the Print button)

How to Use

- Load an SVG file as normal. The spinbox will activate automatically once the file is loaded.

- The spinbox maximum is automatically set to the total number of layers in the loaded file.

- Enter the desired start layer number (minimum: 1).

- Click Print — the printer will begin from the specified layer.

Important Notes

- The spinbox is disabled until an SVG file is loaded. It cannot be edited for bitmap files.

- The entered value is automatically clamped: if a value exceeds the total layer count, it is silently reduced to the last valid layer.

- When starting from a middle layer, ensure that the powder bed has already been manually prepared up to the correct height before pressing Print. The software does not verify physical bed state.

- The current_layer_height is initialized from svg_layer_height[start_layer - 1], so Z-height calculations remain accurate regardless of start layer.

# 3. Print parameter study information

## 3.1 Parameter Configuration (oasis_change_parameters.csv)

| Parameter | Type | Default | Unit | Range | Description |
| --- | --- | --- | --- | --- | --- |
| print_speed | variance | 2200 | mm/min | 100–10000 | Inkjet sweep speed |
| dpi | variance | 300 | dpi | 100–1200 | Print resolution |
| layer_passes | variance | 3 | — | 1–5 | Repeat passes per layer |
| density | variance | 500 | permille | 0–1000 | Ink firing density |
| layer_thickness | variance | 0.1 | mm | 0.05–2 | Layer thickness |
| overfeed_percent | variance | 100 | % | 175–500 | Default (100) is outside valid range — update to ≥ 175 |
| dwell_time | fixed | 5 | s | — | Wait time before camera capture |

## 3.2 Experiment Record Management

Each run automatically saves to runs/<RUN_ID>/:

| File | Contents |
| --- | --- |
| manifest.json | Timestamp, git hash, source CSV path, full resolved config |
| resolved_params.csv | Original CSV values vs. actually applied values (resolved_value column) |

To reproduce an experiment: copy the resolved_value column from resolved_params.csv into the value column of oasis_change_parameters.csv and re-run.